CA-D-317A Safety Guard and LOTO Verification

Controlled field checklist and residual-pressure verification

Checklist reference: SLV-CA317A-2026-06 | Verification date: 2026-06-20 | Package: CA-D-317A

# 1. Scope and control basis

The checklist records package-specific electrical, pneumatic, stored-pressure, heater, condensate, and instrument isolation points. The local point list controls the equipment-specific reconciliation. OSHA 29 CFR 1910.147 is cited as general control-of-hazardous-energy context.

# 2. Current LOTO isolation-point list

| ID | Tag | Isolation point | Energy/source | Verified condition | Evidence |
| --- | --- | --- | --- | --- | --- |
| LOTO-01 | MCC-CA317A | Main compressor motor breaker | Electrical | Locked open | Photo L-01 |
| LOTO-02 | VFD-CA317A | Local enable and control power | Electrical/control | Disabled and tagged | Photo L-02 |
| LOTO-03 | IV-317A | Compressor inlet isolation | Pneumatic | Closed and locked | Photo L-03 |
| LOTO-04 | DV-317A | Package discharge isolation | Pneumatic | Closed and locked | Photo L-04 |
| LOTO-05 | BYP-317A | Dryer bypass isolation | Pneumatic | Closed and locked | Photo L-05 |
| LOTO-06 | HTR-317A | Purge heater breaker | Electrical | Locked open | Photo L-06 |
| LOTO-07 | CD-317A | Automatic condensate drain isolation | Pneumatic | Closed and tagged | Photo L-07 |
| LOTO-08 | PI-DPT317A | Dewpoint instrument impulse isolation | Pneumatic/instrument | Closed and tagged | Photo L-08 |
| LOTO-09 | RV-317A | Receiver outlet isolation | Pneumatic/stored pressure | Closed and locked | Photo L-09 |

# 3. Comparison with older asset-register count

Asset register record AR-017 lists seven points. The older list omitted HTR-317A purge heater breaker and PI-DPT317A dewpoint instrument impulse isolation. The current verified count is nine points.

| Missing tag | Source controlled | Reason absent from old list | Current classification |
| --- | --- | --- | --- |
| HTR-317A | Purge heater electrical supply | Added during heated purge retrofit | Required for current configuration |
| PI-DPT317A | Dewpoint instrument impulse isolation | Added after instrument manifold modification | Required to isolate trapped pressure |

# 4. Residual-pressure verification

| Check | Location | Residual pressure | Unit | Indicator | Condition |
| --- | --- | --- | --- | --- | --- |
| RP-01 | Package discharge header | 0.0 | psig | PG-DIS-317A | After vent and drain |
| RP-02 | Dryer inlet manifold | 0.0 | psig | PG-DRY-IN-317A | After tower equalization |
| RP-03 | Dryer outlet manifold | 0.0 | psig | PG-DRY-OUT-317A | After receiver isolation |
| RP-04 | Dewpoint impulse line | 0.0 | psig | Portable test gauge | After PI-DPT317A closure and bleed |

# 5. Machine-guard clearance and fasteners

| Check | Location/item | Measured | Requirement | Status | Note |
| --- | --- | --- | --- | --- | --- |
| GC-01 | Fan guard lower quadrant | 14.5 | 12.0 | PASS | Smallest measured clearance |
| GC-02 | Fan guard upper quadrant | 18.2 | 12.0 | PASS | No contact marks |
| GF-01 | Guard fasteners installed | 8 of 8 | 8 required | PASS | All fasteners present and secured |

# 6. Access verification

| Check | Item | Access condition | Detail | Status |
| --- | --- | --- | --- | --- |
| ACC-01 | Purge valve PV-317A | West guard panel must be removed | Bolted panel; tool access required | VERIFIED |
| ACC-02 | Prefilter drain | Accessible from south aisle | No obstruction | VERIFIED |
| ACC-03 | Afterfilter bowl | Accessible after receiver isolation | Lift clearance 410 mm | VERIFIED |
| ACC-04 | PSV-317B nameplate | Accessible from east platform | No temporary scaffold required | VERIFIED |

# 7. Guard and access discrepancy history

A thermal inspection note dated 2025-11-04 referred to one absent lower guard fastener. The current controlled checklist and field survey NFS-032 verify that all eight fasteners are installed. The historic observation is superseded for current guard condition.

# 8. Checklist closeout

Current LOTO point count: 9. Minimum measured guard clearance: 14.5 mm against a 12.0 mm local minimum. Residual pressure at all four verification locations: 0.0 psig. No repair recommendation, scheduling instruction, or work assignment is included.